- The Raspberry Pi runs the onboarding AP (IoT-Onboard).

- Devices can onboard dynamically — no preloaded serials.

- A shared onboarding key or admin approval replaces the serial/OTP verification.

- Once onboarded, devices are continuously verified and can be redirected to a honeypot if untrusted.

🧭 Final Project: Secure IoT Onboarding and Identity Management with Honeypot Redirection

🔹 Core Idea

Design a Raspberry Pi–based IoT gateway that automatically:

- Onboards new IoT devices securely (without manual serial setup),

- Assigns them cryptographic identities and certificates,

- Observes their behavior to generate least-privilege policies,

- Continuously evaluates their trust,

- Redirects untrusted or compromised devices to a honeypot for analysis.

⚙️ System Components

| Component | Function |
| --- | --- |
| Raspberry Pi (Main Controller) | Acts as Access Point, Certificate Authority (CA), SQLite identity DB, SDN Policy Engine, and Honeypot Manager. |
| IoT Devices (e.g., SmartTemp-01) | Connect to Pi’s AP for onboarding, get certificates, join main SDN. |
| Ryu Controller | Manages network rules, enforces least-privilege access and redirection. |
| Honeypot Server (Cowrie/Dionaea) | Captures behavior of untrusted devices in isolation. |
| Trust Score Engine | Calculates and updates each device’s trust level based on behavior. |

🪜 Updated Step-by-Step System Workflow

Phase 1 – Initialization

- Start Raspberry Pi Services

  - Enable Access Point (IoT-Onboard) using hostapd.

  - Run dnsmasq for DHCP.

  - Start Flask onboarding service.

  - Load CA certificate and key for signing.

  - Start SQLite database and Ryu controller.

- Database Setup

  - Tables:

    - devices(device_id, mac, cert_pem, trust_score, status, timestamp)

    - policy(device_id, json_policy)

    - events(device_id, type, description, timestamp)

✅ System ready for onboarding.

Phase 2 – Secure Onboarding

- Device Power-On

  - New IoT device (e.g., SmartTemp-01) powers on for the first time.

  - Connects automatically to Pi’s Wi-Fi: IoT-Onboard.

- Device Generates Identity

  - Creates public/private key pair.

  - Generates CSR (Certificate Signing Request).

- Device Sends Onboarding Request

  - Sends HTTP POST to Pi onboarding API:

  - {

  - "device_name": "SmartTemp-01",

  - "csr_pem": "-----BEGIN CERTIFICATE REQUEST-----...",

  - "onboard_key": "PiSecret123"

  - }

- Pi Validates Request

  - Checks for a correct onboard_key (shared local secret).

  - If valid, Pi signs CSR using its CA → generates device.crt.

  - Stores certificate and metadata in SQLite:

  - device_id = auto

  - mac = 192.168.50.20

  - trust_score = 50 (initial)

  - status = "active"

- Pi Sends Response

- {

- "cert_pem": "-----BEGIN CERTIFICATE-----...",

- "ca_crt": "-----BEGIN CERTIFICATE-----...",

- "next_wifi_ssid": "SmartHome_Network",

- "next_wifi_pass": "YourMainWiFiPass"

- }

- Device Switches to Main SDN

  - Leaves IoT-Onboard, joins SmartHome_Network.

  - Uses certificate for TLS-based authentication.

✅ Device identity is now trusted and recorded.

Phase 3 – Behavior Observation and Policy Creation

- Baseline Learning

  - Pi observes network traffic for 20–30 minutes.

  - Collects normal communication endpoints.

- Generate Policy

- Creates least-privilege rules:

- {

- "device": "SmartTemp-01",

- "rules": [

- {"allow": "api.smarttempcloud.com", "port": 443, "protocol": "TCP"},

- {"allow": "time.nist.gov", "port": 123, "protocol": "UDP"},

- {"allow": "DNS", "port": 53, "protocol": "UDP"},

- {"deny": "all_others"}

- ]

- }

- Stores this in SQLite and pushes it to Ryu controller as OpenFlow rules.

✅ SmartTemp-01 now operates under least privilege.

Phase 4 – Continuous Trust Management

- Periodic Trust Evaluation

- The Pi monitors:

  - Behavior vs baseline

  - Network anomalies

  - Attestation responses

- Adjusts trust_score accordingly:

| Event | Score Impact |
| --- | --- |
| Normal operation | +10 |
| Unexpected IP contact | -20 |
| Attestation success | +5 |
| Failed challenge | -30 |

- Policy Update

- Ryu dynamically adjusts access rules based on the current trust score.

✅ Trust-driven dynamic security.

Phase 5 – Detection and Quarantine

- Anomaly Detection

- Device starts sending traffic to unknown IPs.

- Snort/Monitor flags abnormal activity.

- Trust score drops below threshold (e.g., <50).

- Automatic Quarantine and Redirect

- Pi executes:

- redirect_to_honeypot(mac="AA:BB:CC:11:22:33", reason="Anomaly detected")

- Ryu installs redirection flow:

- {

- "dpid": 1,

- "priority": 200,

- "match": {"eth_src": "AA:BB:CC:11:22:33"},

- "actions": [

- {"type": "SET_FIELD", "field": "ipv4_dst", "value": "192.168.40.10"},

- {"type": "OUTPUT", "port": 5}

- ]

- }

- Device traffic now goes to the honeypot server (VLAN 40).

✅ Device safely isolated and monitored.

Phase 6 – Honeypot Analysis

- Honeypot Interaction

- Honeypot logs all device actions:

- [10:45:12] Connection from 192.168.50.20

- [10:45:13] POST /cmd.cgi?passwd=admin

- [10:45:14] Attempted download from 185.53.177.52

- Forensics and Decision

- Admin reviews logs.

- Options:

  - Re-enroll after factory reset.

  - Permanently revoke (certificate blacklisted).

✅ Threat analyzed without impacting main IoT network.

💡 Algorithm Summary (Simplified)

BEGIN

INIT_Pi_AP()

INIT_CA_DB_RYU()

WAIT for device request

IF onboard_key == valid THEN

SIGN_CSR()

STORE_DEVICE_INFO()

SEND_CERT_AND_WIFI()

ELSE

REJECT_REQUEST()

ENDIF

WHEN device joins main SDN:

CAPTURE_TRAFFIC()

LEARN_BEHAVIOR()

CREATE_POLICY()

APPLY_POLICY_TO_RYU()

LOOP every interval:

EVALUATE_TRUST_SCORE()

IF trust_score < 50 THEN

REDIRECT_TO_HONEYPOT()

ENDIF

UPDATE_POLICY()

ENDLOOP

END

🧠 Why This Version Is Better

| Feature | Old Method | New Simplified Design |
| --- | --- | --- |
| Serial preloading | Required | ❌ Removed |
| Manual database entry | Yes | ❌ No |
| Shared onboarding key | No | ✅ Simple & secure |
| User experience | Complicated | ✅ Smooth |
| Security | Strong | ✅ Strong (still Zero Trust) |
| Fits project scope | ✅ | ✅✅ Perfect |

🔐 Final Outcome

After this rearrangement:

- New devices can join easily with a shared key (no serial table).

- Raspberry Pi signs certificates, applies Zero Trust policies.

- Ryu Controller dynamically enforces least-privilege access.

- Trust Engine continuously monitors devices.

- Untrusted devices are automatically isolated and redirected to honeypot.

Your system is now:

💡 “A Zero Trust IoT onboarding and security management platform with automated identity provisioning, dynamic trust scoring, and deception-based containment.”

Would you like me to make a flow diagram (showing the full cycle — Onboard → SDN Join → Monitor → Quarantine → Honeypot → Re-enroll)?
It would visually complete your project presentation and report.

Excellent 👍 — now that we’ve rearranged your project (no serial number preloading, using a shared onboarding key instead), let’s go through a simple, detailed example scenario showing exactly how the system behaves from start to finish — including onboarding, trust management, and honeypot redirection.

You can include this in your report, under sections like
👉 “System Workflow Example,”
👉 “Use Case Scenario,” or
👉 “Demonstration Flow.”

🏠 Example Scenario: SmartTemp-01 IoT Device Onboarding and Honeypot Redirection

System Setup

| Component | Description |
| --- | --- |
| Raspberry Pi Controller | Acts as Access Point (IoT-Onboard), Certificate Authority, Policy Engine (Ryu), and Trust Score Manager. |
| Main SDN Network | SSID: SmartHome_Network — managed by Ryu for traffic control. |
| Honeypot Server | IP 192.168.40.10, placed in VLAN 40 for quarantined devices. |
| IoT Device | SmartTemp-01 (temperature sensor with Wi-Fi). |

🧩 Step-by-Step Scenario Flow

1️⃣ Step 1 – Raspberry Pi Initialization

- The Raspberry Pi starts the following services:

  - hostapd → creates Wi-Fi hotspot IoT-Onboard

  - dnsmasq → assigns IPs (e.g., 192.168.50.0/24)

  - Flask Onboarding Service → listens at http://192.168.50.1:5000

  - Ryu Controller → enforces OpenFlow rules

  - Trust Manager → evaluates device trust

  - Honeypot Redirect Service → handles isolation actions

- The Pi also loads its CA key and prepares its SQLite database.

✅ The Pi is now ready to accept new IoT devices securely.

2️⃣ Step 2 – SmartTemp-01 Joins Onboarding Wi-Fi

- The user powers on the SmartTemp-01 for the first time.

- Since it’s not yet configured, it automatically connects to the Pi’s Wi-Fi SSID IoT-Onboard.

- The Pi assigns it IP: 192.168.50.20.

✅ Device connected to the onboarding gateway.

3️⃣ Step 3 – Device Requests Enrollment

- The device generates a public/private key pair and a CSR (Certificate Signing Request).

- It sends an onboarding HTTP POST request:

POST /onboard

{

"device_name": "SmartTemp-01",

"csr_pem": "-----BEGIN CERTIFICATE REQUEST-----...",

"onboard_key": "PiSecret123"

}

- The shared onboarding key (PiSecret123) ensures only local, authorized devices can onboard.

✅ Basic verification completed.

4️⃣ Step 4 – Pi Verifies and Issues Certificate

- The Pi checks the key — it’s correct.

- Signs the CSR using its own CA private key.

- Creates and stores the certificate in SQLite:

| Device | MAC | Trust | Status | Timestamp |
| --- | --- | --- | --- | --- |
| SmartTemp-01 | AA:BB:CC:11:22:33 | 50 | Active | 2025-10-13 10:25:45 |

- Sends response to the device:

{

"cert_pem": "-----BEGIN CERTIFICATE-----...",

"ca_crt": "-----BEGIN CERTIFICATE-----...",

"next_wifi_ssid": "SmartHome_Network",

"next_wifi_pass": "SmartHome123"

}

✅ Device receives its digital identity and network credentials.

5️⃣ Step 5 – Device Joins Main SDN Network

- The SmartTemp-01 switches to the main Wi-Fi network (SmartHome_Network).

- Uses its new certificate for TLS authentication.

- The Pi verifies the certificate fingerprint before approving SDN access.

- Ryu Controller assigns a “Restricted VLAN” until policy is generated.

✅ SmartTemp-01 is now a verified device on the secure network.

6️⃣ Step 6 – Behavioral Learning Phase

- For the next 20–30 minutes, the Pi observes traffic:

| Destination | Port | Protocol | Notes |
| --- | --- | --- | --- |
| api.smarttempcloud.com | 443 | TCP | Sends sensor data |
| time.nist.gov | 123 | UDP | Gets time updates |
| 8.8.8.8 | 53 | UDP | DNS queries |

- From this, Pi learns the normal traffic baseline.

✅ Device shows clean, expected behavior.

7️⃣ Step 7 – Least-Privilege Policy Creation

- Pi generates a JSON policy and sends it to Ryu:

{

"device": "SmartTemp-01",

"rules": [

{"allow": "api.smarttempcloud.com", "port": 443, "protocol": "TCP"},

{"allow": "time.nist.gov", "port": 123, "protocol": "UDP"},

{"allow": "DNS", "port": 53, "protocol": "UDP"},

{"deny": "all_others"}

]

}

- Ryu translates this into OpenFlow rules.

- Any packet not matching these rules is automatically dropped.

✅ The device now runs under least privilege.

8️⃣ Step 8 – Trust Scoring and Attestation

- Pi assigns trust_score = 80 after successful onboarding and stable behavior.

- Periodically, the Pi challenges the device:

- Challenge: Nonce = 891XYZ

- Device signs it with its private key and sends back the signature.

- Pi verifies it → valid ✅ → trust remains stable.

✅ Trust management running continuously.

9️⃣ Step 9 – Anomaly Detected

After a few days, the SmartTemp-01 device gets compromised.

- It starts sending suspicious requests:

- POST http://185.53.177.52/botnet.php

- IDS (Snort) flags it as malicious traffic.

- Pi detects it violated its baseline.

- Trust score drops to 45 (below threshold).

✅ Device marked as Untrusted.

🔟 Step 10 – Automatic Honeypot Redirection

- Pi runs:

- redirect_to_honeypot(mac="AA:BB:CC:11:22:33")

- Ryu installs redirection rule:

- {

- "dpid": 1,

- "priority": 200,

- "match": {"eth_src": "AA:BB:CC:11:22:33"},

- "actions": [

- {"type": "SET_FIELD", "field": "ipv4_dst", "value": "192.168.40.10"},

- {"type": "OUTPUT", "port": 5}

- ]

- }

✅ All SmartTemp-01 traffic now flows to the honeypot (192.168.40.10) in Quarantine VLAN.

1️⃣1️⃣ Step 11 – Honeypot Interaction

- Honeypot receives and records all traffic:

- [11:12:03] Connection from 192.168.50.20

- [11:12:04] POST /cmd.cgi?user=admin&pwd=1234

- [11:12:05] Attempted download from 185.53.177.52

- The honeypot safely captures the malicious payloads.

- Pi logs this in its SQLite events table for analysis.

✅ Attack contained — rest of the network stays safe.

1️⃣2️⃣ Step 12 – Admin Review & Recovery

- Admin opens Pi dashboard:

| Device | Trust | Status | Redirected | Last Event |
| --- | --- | --- | --- | --- |
| SmartTemp-01 | 45 | Quarantine | Honeypot | Anomaly detected |

- Admin inspects logs and confirms compromise.

- Options:

  - Factory reset device and re-onboard, or

  - Revoke certificate permanently.

✅ Security breach handled automatically.

🧠 Outcome Summary

| Phase | Action | Result |
| --- | --- | --- |
| Initialization | Pi launches AP, CA, Ryu, Honeypot | System ready |
| Onboarding | Device requests cert with shared key | Identity issued |
| SDN Join | Device joins main network | Verified access |
| Learning | Pi records normal behavior | Baseline created |
| Trust Evaluation | Trust score managed dynamically | Continuous security |
| Anomaly Detection | Suspicious activity found | Score drops below threshold |
| Redirection | Device isolated to honeypot | Threat contained |
| Analysis | Admin reviews captured traffic | Attack studied safely |

💬 Final Summary for Report

In this demonstration, a new IoT temperature sensor securely onboarded through the Raspberry Pi’s Wi-Fi without any manual serial registration.
The Pi automatically issued a certificate, applied Zero Trust policies, and continuously monitored the device’s behavior.
When the sensor later showed abnormal behavior, the system automatically reduced its trust score and redirected it to a honeypot for safe observation.
This ensures the entire IoT network remains secure even when individual devices become compromised.

Would you like me to draw a system flow diagram for this scenario — showing each stage (Onboard → Policy → Trust → Honeypot) with arrows and components labeled (Device, Pi, Ryu, Honeypot)?

It’ll make this section visually perfect for your slides or report.